§ 6.03 SUMMARY TABLE OF DIMENSIONAL STANDARDS.

The following table offers examples of the dimension standards for several zoning districts. The Agricultural Interpretive Center District (AIC) and Shoreland Standards are not included in this table. Please refer to the dimension standards included in the individual zoning districts as well as other provisions, standards, regulations, requirements as contained in the Unified Development Code, including, but not limited to, the development review processes and requirements, general regulations, general environmental regulations, conservation design, transfer of development rights and the like.

| Table 6.2 | Agricultural A-1 | Limited Residential LR | Urban Expansion UE | Village Mixed Use VMX | Highway Commercial HC | General Industrial I |
| --- | --- | --- | --- | --- | --- | --- |
| Minimum lot area | 1 ac min, 5 ac max for sfd, 3 ac min1 | Standard Lots: 1 ac Conservation Lots: 0.75 ac; 2.5 ac max for sfd 2.5 ac min1 | 1 ac for sfd 2.5 ac1 for other principal structures | 30,000 sqft2 1 ac1 | NA | NA |
| Minimum lot width | 100ft2 | Standard Lots: 150ft Conservation Lots: 75ft | 100ft | 75ft | 100ft | 100ft |
| Front yard setbacks | 75ft | Standard Lots: 40ft Conservation Lots: 30ft from interior road All lots - *50ft in SO; **25ft from private road | 50ft | 35ft3 | 50ft | 50ft |
| Windbreak or tree | 20ft4, 15ft5 | 20 ft4, 15 ft5 | 20ft4, 15ft5 | 20ft4, 15ft5 | 20ft4, 15ft5 | 20ft4, 15ft5 |
| Side yard setbacks | 40ft | Standard Lots: 15ft Conservation Lots: 10ft | 20ft | 20ft | 15ft6 | 15ft6 |
| Tree | 20ft | 20 ft | 20ft | 20ft | 20ft | 20ft |
| Shrub | 10ft | 10 ft | 10ft | 10ft | 10ft | 10ft |
| Rear yard setbacks | 40ft | Standard Lots: 30ft Conservation Lots: 25ft | 30ft | 30 ft | 15ft6 | 15ft6 |
| Tree | 20ft | 20 ft | 20ft | 20ft | 20ft | 20ft |
| Shrub | 10ft | 10 ft | 10ft | 10ft | 10ft | 10ft |
| Accessory structure setbacks | 75ft Side & Rear: 20ft* | Front: Standard Lots: 40ft Conservation Lots: 30ft from interior road All lot - *50ft in SO; **25ft from private road Side & Rear: 15ft | 50ft Side & Rear: 20ft (10 ft if structure ≤100) | 35ft Side & Rear: 20ft (10 ft if structure ≤100) | 50ft Side & Rear: 20ft (10 ft if structure ≤100) | 50ft Side & Rear: 10ft |
| Maximum impervious coverage | None required | 25% (outside Shoreland District) | 25% | 50% | 65% | 75% |
| Maximum height | 35ft | 35ft | 35ft | 35ft | 35ft | 70ft |
| 1Minimum lot size for permitted and conditional uses other than a single-family dwelling |  |  |  |  |  |  |
| 2County ISTS standards must be met |  |  |  |  |  |  |
| 3Or within range of existing setbacks |  |  |  |  |  |  |
| 4When public right-of-way is less than 100 feet in width |  |  |  |  |  |  |
| 5When public right-of-way is greater than 100 feet in width |  |  |  |  |  |  |
| 6The yard setback is 50 feet when adjacent to a residential or agricultural district |  |  |  |  |  |  |
| 7Agricultural buildings are exempt from height limitations, non-Ag buildings not to exceed 2.5 stories or 35 feet height, whichever is greater |  |  |  |  |  |  |
| *An exemption to the Side & Rear yard setback requirement may be given for two (2) contiguous parcels, in the A-1 Agricultural Protection Zoning District, under one ownership or owned by a family member based upon the definition of “FAMILY” provided the owner(s) and family member(s) signs an affidavit that will be recorded stating that they have no objection to an agricultural structure being closer than the required 20ft. All other requirements of this ordinance shall be adhered to. |  |  |  |  |  |  |
| **Minimum front yard setback when a property is accessed via a private road |  |  |  |  |  |  |